# AUTOCOOKIE user manual

Homework Tips Checklist for Parents

AutoCookie allows you to turn a clip art image into a cookie cutter

# Prepare your image

☐	Find a clipart image with a white background that you want to become your cookie cutter.

☐   Download and name this file “candidate.jpg” and put it onto the AutoCookieSoftware folder that is given to you on the flash drive.

☐	Run the AutoCookie application in the folder.

☐	Once the file is generated (~100s) it will be saved as “finished_prod.jpg.”

# File conversion

☐	Go to https://convertio.co/jpg-svg/ and convert finished_prod.jpg to an svg file. Download your converted file.

☐	Go to http://svg2stl.com/ and select your new svg file.

☐	Click upload. Extrude 35 mm. Finish conversion and download your stl file.

# Print your cutter

☐	Download Utimaker Cura (newest version).

☐	Upon first launch you’ll want to add a new printer:

- Navigate to Settings > Printers > Manage Printers

- Click add, then add non-networked printer

- Click custom printer

- Click on custom FFF printer #1 then machine settings

- Under Printer Settings: Change X (Width), Y (Depth) and Z (Height) to 180 mm, Heated Bed should be checked.

- Under Printhead Settings: X min = -20mm, Y min = -20mm, X max = 20mm, Y max = 20mm, Gantry Height = 180 mm.

- Go to the extruder 1 tab and change compatible material diameter to 1.75 mm and Cooling Fan Number to 1.

- Click next and you’re done!

☐	To the right of your printer you should see the material selection. Select Generic PLA.

☐	To the right of the material is print settings. For your first print you’ll want to change some settings:

- Under Profile, select Draft-2.0mm

- Under Material, Printing Temp = 200C and Build Plate Temperature = 70C

- Under Speed, set Print Speed to 50mm/s.

- Under Cooling, make sure Enable Print Cooling is checked, Fan Speed 100%.

- Under Build Plate Adhesion Type, select Brim.

☐	You’ll likely want to rescale to the size you want. The second option on the left toolbox is the rescaling tool. Rescale to a percentage of your original size; cura gives you the XYZ dimensions to help.

☐	Click Slice and upload to a microSD card

☐	Put the microSD into the Kingroon Printer, find the file and begin printing!